# Who Crosses and When: What Event-Anchored Benchmarks Measure in Pedestrian Crossing Prediction

### Anonymous Authors

Aflliations withheld for blind review

Keywords: Pedestrian Crossing Prediction, Streaming Evaluation, Benchmark Protocols, Survival Analysis, Cure Models, Intelligent Vehicles

## Abstract

Pedestrian crossing prediction, fundamental to safe autonomous driving, is usually benchmarked with an event-anchored protocol with a few windows per pedestrian, one to two seconds before a reference moment, are labelled by whether the pedestrian crosses the road ahead. However, in actuality, a decision system has to determine this pedestrian crossing behavior at every moment. In this paper, we show that the streaming label has two parts: whether someone will cross in front of the vehicle at all and, if so, when the crossing starts. The anchored protocol ignores the latter. On the Pedestrian Intention Estimation (PIE) dataset, models trained under the anchored protocol competently rank who crosses, but their ranking of when crossings start is at or below chance. For a neural network, the area under the receiver operating characteristic curve drops from 0.89 on anchored to 0.52 on streaming test data, and for boosted trees, from 0.95 to 0.47. Threshold retuning and re-weighted losses do not salvage this. Motivated by that, we used scene information from PIE annotations in the decision system. We observe from our experiments that, doing so raises the who score from 0.755 to 0.853, above a human intention rating (0.739; a post hoc comparison), but no input we added raises the when score by more than 0.043. With the same training windows, predicting crossing time does no better than binary training.

## Introduction

A pedestrian-crossing detector on a moving vehicle must make a prediction at every moment for every pedestrian in view. Most of those moments do not precede a crossing. The standard evaluation protocol asks a narrower question. Introduced with the Pedestrian Intention Estimation (PIE) and Joint Attention in Autonomous Driving (JAAD) datasets [1, 2], and later standard-ised in an evaluation suite [3], it samples a few windows from each pedestrian track. These windows end one to two seconds before a reference moment and are labelled according to whether the pedestrian crosses (Figure 1a).

Interpreting a score from this protocol requires separating two questions. The first is who: will the pedestrian cross in front of the vehicle at all? The second is when: if the pedestrian will cross, will the crossing begin within the next second? A streaming detector has to answer both. It should remain silent, for example, when a pedestrian will cross only five seconds later (Figure 1b). We show that the streaming label is the product of these two components. The anchored protocol holds the second component fixed and therefore scores only the first. A single anchored score cannot reveal which component makes the streaming task difficult.

We trained a neural network and boosted trees on the same pose and kinematic inputs. We then added scene, behaviour and oracle inputs to the trees, alone and combined, and the scene inputs also to the network; the oracles provide information unavailable to a deployed system. Each study was specified in writing and time-stamped before its first result. The study makes five contributions:

- Decomposition. We factor the streaming crossing label into who and when, corresponding to incidence and latency in a mixture cure model from survival analysis. The anchored label estimates who alone (Section 3).

- An explanation of the protocol gap. In both model families, anchored training ranks who well but reverses the ordering of when. The area under the receiver operating characteristic curve (AUC) falls to chance on streaming data. Neither threshold retuning nor re-weighted losses remove this gap, and the smaller size of the anchored training set does not explain it (Section 5).

- Decomposed evaluation. We score who and when sep-arately and count false alarms both per window and per pedestrian. These two ways of counting lead to different conclusions (Section 6).

- An information audit. Scene information raises who above a human intention rating, although that comparison is post hoc. When improves by at most 0.043, less than half the gain in who (Section 6).

- A timing objective. On the same training windows, an objective that models time to crossing does no better than binary training in either model family. This agrees with the decomposition if the inputs contain no timing information beyond what binary training already uses (Section 7).

Streaming evaluation is established in online action detection [4, 5], and traffic research has distinguished immediate crossing from waiting time (Section 2). However, to our knowledge and based on a targeted search, no previous study of crossing prediction from vehicle video separates who from when or shows that the anchored score measures only who.

## Related Work

  - Crossing Prediction on PIE and JAAD

Most studies using PIE [1] and JAAD [2] follow the benchmark of Kotseruba et al. [3]. They assign labels per track and use an observation period ending one to two seconds before the

    - Event-anchored

a few windows, label C: who only

crossing onset

| never sampled |  |  | never sampled |
| --- | --- | --- | --- |

1-2 s

    - Streaming

at-risk windows: label y = who (C = 1) × when (onset within H)

crossing onset

horizon H

positive: onset within H   hard temporal negative: onset beyond H   crossing in window (discarded)   already crossed (label 0)

Figure 1 The two sampling protocols on one pedestrian track. (a) Anchored windows end a known interval before the crossing and carry the label 𝐶 (does the pedestrian cross), which scores who only. (b) Streaming windows cover the whole track, and their label combines who and when. Windows are drawn further apart than in practice, where they are 20 frames long and start every 3 frames.

crossing. Several studies have questioned what the resulting scores measure. Bounding boxes alone can yield high accuracy on PIE [6], predictors transfer poorly between datasets [7], and models depend heavily on ego-vehicle speed, which may reflect the driver’s behaviour rather than the pedestrian’s [8].

Rasouli and Kotseruba [9] point out that intention and action are often used interchangeably. They also report much higher action-prediction scores when the ego vehicle is stationary, a setting in which most samples are crossers. Kotseruba et al. [10] provide a human reference for assessing a pedestrian’s intention to cross. Yao et al. [11] report results for both whole tracks and an “event of crossing” setting restricted to the one to two seconds before crossing. Most of these studies retain the window population of the anchored protocol, and none separates the label into who and when. Our evaluation changes both the sampled windows and the label.

  - When Pedestrians Start to Cross

Early work on path prediction found that a pedestrian approach-ing the kerb could be reliably classified as stopping or crossing only a fraction of a second before the event [12]. Later work incorporated context, including awareness of the vehicle and distance to the kerb, to predict transitions between walking and standing [13]. More recent studies predict whether a pedestrian will start or stop walking within two seconds [14], collect frame-by-frame intent labels from many annotators [15], and report crossing-prediction performance against time to the event [16]. These studies model timing or vary it in evaluation. We show that the standard score is insensitive to it.

  - Online Action Detection

Online action detection assigns a label to each frame in a stream using only present and past evidence [4]. In the action-start variant, the system must signal the beginning of an action as early as possible [5]. We adopt this streaming setting and its approach to counting false alarms.

  - Time-to-Event Modelling

Survival analysis models time to an event when some observa-tions end before the event occurs [17]. Mixture cure models separate whether an event occurs from when it occurs [18, 19], giving the structure used in Section 3.4. In traffic research,

survival and duration models have been used to describe pedes-trians’ waiting times at the kerb [20, 21]. Yang et al. [22] combine a logit model for pedestrians who cross immediately with a duration model for those who wait, a separation close to ours. Kalatian and Farooq [23] fitted a neural survival model to waiting times in virtual reality. We use the cure structure to interpret an existing vision benchmark and use a hazard objective for time to crossing as a further check (Section 7).

## Problem Formulation

  - Event-Anchored Sampling

For a pedestrian tracked with annotated frames 1, . . . , 𝑛, our implementation of the anchored protocol [3] takes up to six windows of 𝑇 frames, whose last frames lie 30 to 60 frames (one to two seconds) before a reference frame annotated by PIE. For a pedestrian who crosses, this reference frame is the start of the crossing; for a pedestrian who does not cross, this is the last frame in which the pedestrian is visible [3]. All windows extracted from a track receive the same track-level label (whether the pedestrian crosses or not), therefore the class ratio reflects how many crossing (positive) and non-crossing (negative) tracks were sampled. In our PIE’s test set, the ratio is 1 positive sample for every 2.6 negative samples.

  - Streaming Sampling

In our streaming protocol, a pedestrian track is sampled begin-ning from the first frame, and window are extracted every few frames along the whole track. The window ending at frame 𝑡 is labeled:

𝑦𝑡 = 1 [ ∃ onset in (𝑡, 𝑡 + 𝐻] ] ,	(1)

where 𝐻 is the anticipation horizon. Windows in which the pedestrian is already crossing are discarded. The positive class ratio is in this case the ratio of windows in which a crossing is about to begin. In the PIE dataset, this accounts for 2.9% of the training windows, or one positive sample every 33.9 negative samples. Despite the seemingly obvious significant imbalance, the two ratios measure different quantities, rather than one task being outright more difficult than the other.

  - Three Kinds of Streaming Negative

The streaming protocol produces three types of negative sample (Figure 1b):

- Never-crosser windows: pedestrians who wait or walk alongside the pavement and do not cross on camera. These pedestrians are also sampled by the anchored protocol.

- Hard temporal negatives: pedestrians whose crossing starts at more than 𝐻 after the window ends. These windows are labeled negative, however they show the same person with near identical features with the positive windows that follows, and only the position of the cutoff give them a different label

- Already-crossed: pedestrians who have finished crossing, which are labeled 0 like never-crosser.

Only the first kind of negative sample is produced by the anchored protocol. Windows of any kind may also be uninformative, e.g., when the pedestrian is occluded or too far away.

  - The Streaming Label Factors into Who and When

The streaming label of Equation (1) combines two questions: whether the pedestrian will cross at all, and whether the crossing starts within the horizon. For a window ending at frame 𝑡, let 𝑥

Table 1 Composition of the streaming windows in each split (%). The “will cross later” column holds the hard temporal negatives, which the anchored protocol does not generate.

| Split | positive | will cross later | already crossed | never-crosser |
| --- | --- | --- | --- | --- |
| train | 2.9 | 25.7 | 7.2 | 64.3 |
| val | 2.8 | 15.2 | 4.6 | 77.5 |
| test | 3.1 | 39.2 | 4.3 | 53.4 |

The anchored protocol estimates only who. Its label is 𝐶, which records whether the pedestrian crosses but not when the crossing starts. Its sampling also holds when nearly fixed, as every window ends 30 to 60 frames before the reference frame. A model can therefore reach a high anchored score even if it orders imminent and distant onsets arbitrarily.

Both factors are scored with an AUC of the model’s crossing score 𝑠𝑡 , so no extra output is needed. For who, each pedestrian is represented by the mean 𝑠𝑡 over their at-risk windows, and crossers are ranked against never-crossers. For when, only the windows of crossers are used: those whose crossing starts within

𝐻 are ranked against those whose crossing starts later (hard temporal negatives). A value below 0.5 means that timing is ranked backwards. Because this score pools the windows of all

denote what the model observes (the last

𝑡

𝑇 frames). Let 𝜏 be

crossers, it can also be shifted by how high a model scores each

crosser overall, so we also report it averaged within each crosser

the frame at which the pedestrian’s next crossing in front of the vehicle begins (𝜏 = ∞ if none follows before the track ends). We write 𝐶 = 1[𝜏 < ∞] for the indicator that the pedestrian crosses at all. A window is at risk unless it lies after the pedestrian’s last crossing. On at-risk windows, 𝑦𝑡 = 1[𝜏 ≤ 𝑡 + 𝐻], and a window can only be positive if the pedestrian crosses (𝐶 = 1),

so the chain rule gives

𝑃(𝑦𝑡 =1 | 𝑥𝑡 ) = 𝑃(𝐶=1 | 𝑥𝑡 ) 𝑃(𝜏 ≤ 𝑡+𝐻 | 𝐶=1, 𝑥𝑡 ) . (2)

who

when

x_________˛z_________x x______________________˛z	x

(Section 6.3).

The decomposition implies three consequences, which we label P1 to P3 and compare with the results of later sections:

P1 A threshold does not change the order of the windows, so retuning it cannot correct a reversed when score and should recover almost none of the anchored-to-streaming gap (Section 5.2).

P2 On the same training windows, an objective that models the time to the crossing can beat binary training only if the

The who factor is the probability that the pedestrian crosses at all, which differs from intent: a pedestrian who intends to cross may not cross while the vehicle is present (Section 6.2). The when factor is the probability that a crosser starts within the horizon. Already-crossed windows keep the label 0; they count as negatives in window-level metrics but are excluded from the scores below and from the detection curve (Section 5.1).

Equation (2) has the structure of the mixture cure model of survival analysis [18, 19], which separates whether an event occurs (its incidence, here who) from when it occurs (its latency, here when). Unlike standard survival models, it allows subjects who never have the event, called cured. This fits our setting, as most pedestrians in PIE never cross in front of the vehicle (Section 5.1). We use the model only to interpret the label and do not fit it to our data. First, survival models can learn from subjects whose observation ends before the event. In our data, such subjects are the windows whose future was not recorded (censored windows), which a binary label must drop or set to 0 (Section 7). Second, a subject whose observation ends before any event cannot be distinguished from a cured one. A track that ends without a crossing cannot show whether the pedestrian would have crossed later, so 𝐶 is defined on the observed track only (Section 6.4).

inputs carry timing information that binary training leaves unused (Section 7).

P3 Information that improves only who should raise detection more when false alarms are counted per pedestrian than per window (Section 5.1). When alarms are counted per window, a few never-crossers who wait near the road for a long time can still use up the whole budget (Section 6.4).

All three results were measured before the decomposition was written down; they agree with it but were not pre-registered tests.

## Composition of the Streaming Windows

We use the standard PIE split, with sets 01, 02 and 06 for training, 04 and 05 for validation and 03 for testing. Streaming windows have 𝑇 = 20 frames, start every 3 frames and use 𝐻 = 32 frames (1.07 s at 30 fps). Censored windows, whose future is only partly observed and shows no crossing, are dropped instead of being labelled negative, since a crossing may have happened in the unrecorded part. This leaves 88 214 training, 20 490 validation

and 69 875 test windows, with 2.9, 2.8 and 3.1% positives. Hard temporal negatives make up 25.7% of the training windows and 39.2% of the test windows (Table 1). Most hard temporal

negatives lie far from the horizon: in the training split, the onset is more than five seconds ahead for 68.7% of them. We keep 𝐻 short, because a longer horizon drops more censored windows, and the hard cases near the cutoff only move with it (Section 7.2).

## The Protocol Gap, Explained

Table 2 Cross-protocol evaluation of the crossing task. Cells: AUC · F1 score at threshold 0.5 → validation-tuned F1, mean over seeds (neural network 3, trees 5). Bold: anchored-trained models on streaming test data.

test protocol

Model  trained on	anchored	streaming

  - Experiment Set Up

neural net

anchored 0.889 · 0.730 → 0.729 0.516 · 0.063 → 0.062

streaming 0.696 · 0.388 → 0.459 0.783 · 0.237 → 0.243

We use two learners for the evaluation of this task: a temporal

boosted anchored  0.948 · 0.834 → 0.830 0.471 · 0.062 → 0.062

neural-network and a gradient-boosted tree. Both learners see the same input: a 20-frame window per sample, with each frame being a 58-dimensional vector of 49 pose measurements from 23 whole-body keypoints estimated with DWPose [24] and 9

trees	streaming 0.707 · 0.412 → 0.488 0.821 · 0.281 → 0.294

channels for bounding box position, size, velocity, and ego-vehicle speed. The neural network standardizes each channel, encodes the window with two temporal convolutions, a two-layer gated recurrent unit and multi-head self-attention of width 128 (0.66 M parameters), and pools per-frame crossing scores with log-sum-exp. It was trained with a sampler that oversamples crossing windows, a learning rate of 10−4 and at most 30 epochs, and selected on validation AUC, with 3 seeds. The boosted trees summarize each channel by its last value, mean, change, and standard deviation over the window (232 features). They were fitted by gradient boosting [25] in scikit-learn with 300 iterations and 31 leaves, cut at the best validation iteration, with 5 seeds. On this input the trees detect more crossers than the neural network and vary less between seeds, which makes comparisons between tree models more precise, although we do not claim that trees are better models in general.

Detection curve. A driver needs one timely warning for each crossing pedestrian at a tolerable number of false alarms, which window-level scores such as AUC and F1 do not measure. We therefore evaluate every model with a detection curve. Each test window is one decision (whether to raise an alarm), so one hour of one pedestrian’s footage gives 36 000 decisions. A crossing pedestrian counts as detected if any of their windows before the onset scores above the threshold, and the earliest such window gives the lead time (how long before the onset the first warning came). Alarms raised after the onset are ignored.

Instead of fixing the threshold, we fix a false-alarm budget (false alarms per hour of footage) of 1200, 460, 205, 95 or 41. As in quickest change detection [26], each budget sets the lowest threshold whose alarm rate stays within it, so a model cannot gain detections by raising more alarms.

False alarms are counted per window or per pedestrian, and every number names its count. Per window, 205 alarms per hour lets 0.57% of never-crosser windows raise an alarm. Per pedestrian, a never-crosser who raises any alarm is counted once, so a pedestrian who waits at the kerb for a long time costs one alarm instead of many. The test split holds 205 crossing and 495 never-crossing pedestrians, with 37 316 never-crosser windows (1.04 hours of footage), so at 205 alarms per hour up to 212 never-crossers (43%) may raise an alarm. At the loosest budget, every model detects all crossers when counted per pedestrian, therefore the per-pedestrian verdicts in Section 6 use the other four budgets.

Decision rule. With only 205 crossing pedestrians in the test split, a detection rate depends on which pedestrians happen to be

in it. Before each study, we therefore fixed one rule for deciding whether a gap between two models is larger than chance, and every detection comparison below follows it unless marked post hoc.

The rule uses a paired bootstrap [27]. We draw pedestrians from the test split at random with replacement, score both models on the same draw, and record the gap between their detection rates. Seeds are drawn in the same way. Pedestrians are drawn rather than windows, because the windows of one pedestrian overlap and are not independent. After 2 000 draws, the middle 95% of the gaps forms the 95% interval. If the interval excludes zero, one model was ahead in almost every draw.

A model is called better if its interval excludes zero at 3 or more of the five budgets [reason for 3 of 5]. The two models are called equivalent if the interval lies within ±5 points at 3 or more budgets [28], about the smallest gap our test split can

resolve (Section 8.3). Otherwise the result is inconclusive: the gap did not clear the bar, which does not mean the models are equal. The per-window count is the primary one. For the neural networks, whose runs raise barely correlated alarms, we use a simpler rule: a gap between seed means larger than the sum of their standard deviations, at 3 or more budgets.

  - The Cross-Protocol Matrix

In Table 2, each model was trained under one protocol and tested under both anchored and streaming test set. Neural model and tree trained on the anchored protocol both scored well on anchored test data (0.889 and 0.948 AUC, 0.730 and 0.834 F1 score) and fell to blind-guessing or inverse signal on streaming test data (0.516 and 0.471 AUC, 0.063 and 0.062 F1 score). Retuning the decision threshold does not rescue the F1 score, as expected from P1 (Section 3.4) and from the low AUC score. Focal loss [29] and class-balanced weights [30] was also tested for the anchored trained neural network, which scores an AUC of 0.545 and 0.539, below the pre-registered bar of 0.60, and

the gain from retuning remained at −0.006 and +0.001.

It can also be pointed out that the anchored training set has 4 872

windows, while the streaming training set is 18 times larger due to sequence generation logic. However, a tree trained on the same number of randomly drawn streaming windows still detected 44.2% of crossing pedestrians at 205 alarm-per-hour budget, compared with 22.1% for the tree trained under anchored protocol. It also scored an AUC of 0.779 compared to 0.471 (Table 3); therefore the collapse after of anchored trained models cannot be attributed to the size of the training set.

Table 3 Window AUC, who and when on the streaming test split, with detection rate (%) at 205 alarms per hour, counted per window. Who and when: mean ± standard deviation over

seeds (neural network 3, trees 5). ∗Trained on as many

streaming windows as the anchored training set holds.

| Model | trained on | AUC who | when det. |
| --- | --- | --- | --- |
| neural net | anchored | 0.516 0.799 ± 0.010 | 0.414 ± 0.012 15.1 |
| neural net | streaming | 0.783 0.766 ± 0.034 | 0.753 ± 0.008 42.3 |
| trees | anchored | 0.471 0.839 ± 0.006 | 0.356 ± 0.010 22.1 |
| trees | streaming | 0.821 0.755 ± 0.005 | 0.805 ± 0.003 51.4 |
| trees | streaming∗ | 0.779 0.710 ± 0.021 | 0.776 ± 0.007 44.2 |

  - Who and When

In Table 3, the same model and tree are scored on the two factors of Section 3.4. Both of them when trained under the anchored protocol ranked the who question well at 0.799 and 0.839 AUC, while the when question received inverse signal answer at 0.414 and 0.356 AUC. This means that among all crossing pedestrians windows, those far from onset are scored higher by both models than those nearer to it. When averaged within each crossing pedestrian, when moved closer to blind-guessing at 0.477 and 0.438 AUC (post hoc). Models trained under streaming protocol scored lower on who (Table 3) but significantly higher on when. This is partly because who averages a crosser’s scores over all their window, and a model that has learned timing scores windows further from the onset lower. Separation from never-crosser also rose with time-to-onset for anchored trained models and vice versa for streaming trained models (Figure 2). This pattern is expected from the anchored sampling, where positives windows are fixed 30 to 60 frames before the crossing and the last moments before a crossing never appears in anchored training. Window AUC is the share of positive-negative window pairs that a model ranks correctly, so it averages over our 3 distinct kinds of negative sample. Hard temporal negatives make up 40.5% of the test negatives, and against these the AUC is reflected through the when score. Never-crosser windows make up 55.1% of test negatives and for these, the anchored model and tree ranked positives higher in only 60.1% and 54.9% of pairs. Together, these two comparisons bring window AUC close to blind-guessing chance.

## Where the Difficulty Lies

  - The Information Audit

To analyze the influence of input features on the who and when factors, we add input groups to the pedestrian-only streaming protocol tree, each on its own and combined (Table 4), and kept every other setting fixed. The three groups are lights (state of the largest visible vehicle traffic light and pedestrian signal), traflc (number of vehicles and distance to the nearest), and layout (intersection type, lanes, signals, crosswalk distance). We also test two non-deployable oracles: PIE’s human intention rating (give the pedestrian scene to a real human and ask whether the pedestrian will cross), and the ego-vehicle’s speed change over the next second, which is confounded, as drivers slow down near crossing pedestrians. All features are available from PIE annotations.

Table 4 Information audit on the streaming test split (trees, mean over 5 seeds): who, when, window AUC and detection rate (%) at 205 or 41 alarms per hour, counted per window (pw) or per pedestrian (pp). †Oracle, not deployable. ‡PIE’s human rating alone, per pedestrian (post hoc). §Neural network, 3 seeds.

detection

Input	who when AUC pw 205 pp 205 pp 41

| pedestrian only | 0.755 | 0.805 | 0.821 | 51.4 | 86.0 | 52.3 |
| --- | --- | --- | --- | --- | --- | --- |
| + lights | 0.778 | 0.833 | 0.842 | 50.7 | 86.4 | 54.5 |
| + traffic | 0.791 | 0.796 | 0.818 | 50.6 | 86.9 | 52.2 |
| + layout | 0.819 | 0.829 | 0.851 | 55.0 | 87.7 | 65.0 |
| + scene (all three) | 0.853 | 0.845 | 0.866 | 52.7 | 92.3 | 62.6 |
| + waiting history | 0.758 | 0.811 | 0.829 | 47.8 | 88.0 | 49.7 |
| + scene, waiting, looking | 0.855 | 0.848 | 0.873 | 53.1 | 92.4 | 67.3 |
| + intent oracle† | 0.845 | 0.792 | 0.843 | 59.8 | 93.4 | 61.5 |
| + ego-future oracle† | 0.788 | 0.809 | 0.830 | 52.8 | 89.6 | 55.7 |
| + scene, both oracles† | 0.892 | 0.835 | 0.881 | 53.9 | 97.9 | 66.2 |
| neural net, pedestrian only§ | 0.766 | 0.753 | 0.783 | 42.3 | 86.3 | 44.6 |
| neural net + scene§ | 0.854 | 0.791 | 0.826 | 32.5 | 94.0 | 56.4 |
| human intention rating‡ | 0.739 | – | – | – | – | – |

  - The Question of Who Improves with Scene Information

With scene information, who rose from 0.755 to 0.853 (Table 4). This is above the human intention rating, which separated crossers from never-crossers at 0.739 (a post hoc comparison). Most of the gain came from layout (0.819 alone), while lights and traffic alone added less (0.778 and 0.791). That a model can rank who better than the rating is expected, since the rating estimates intent (Section 3.4): 373 of the 495 never-crossers (75%) were rated at least 0.5 (a post hoc count). Counted per pedestrian, detection rose by +6.3, +10.7 and +10.3 points at 205, 95 and 41 alarms per hour (pre-registered secondary count: better). Counted per window, the primary count, the gain of

+1.3 to +6.2 points was inconclusive.

  - The Question of When Improves Little

The gain in when was much smaller. With scene information, it rose by 0.040, less than half the increase in who (0.098), and no input group or combination raised it by more than 0.043. Averaged within pedestrians (191 crossers), when increased from 0.792 to 0.832 (post hoc). In a second pre-registered study, the neural network received the scene groups as extra input at every frame; its who rose from 0.766 to 0.854 and its when from 0.753 to 0.791, again less than half as much (Table 4).

In the pre-registered test for when, we added the pedestrian’s waiting history at the kerb, which is regularly modelled in traffic studies [20, 21, 22]. The resulting gain of +0.006 counted as no gain under the pre-registered rule, which requires a gain of at least 0.02 with a 95% interval that excludes zero. Similarly, looking history added +0.003, and both histories together also added only +0.003. The human intent oracle carries no timing information: it raised the who score while when fell slightly to 0.792, and the ego-vehicle’s speed oracle barely raised it by

+0.004.

Separation from never-crosser and the when score both depend heavily on how far ahead the crossing is. Within a second of onset, all streaming trained models across various input groups addition ranked positive windows above never-crosser windows

AUC against never-crossers

0.9

0.8

0.7

0.6

0.5

trees, pedestrian only trees, + scene (all three) trees, + scene + oracles neural net, streaming trees, anchored-trained

neural net, anchored-trained

chance

0-1	1-2	2-3	3-5	5-10	≥ 10

time remaining until crossing onset (s)

at 205 per hour came from fewer never-crossers (39.4 to 25.4, mean over seeds). However, the three never-crossers with the most alarms held more of these windows (63.6 and then 78.2). For the neural network, the alarms came from 35 and then 15 never-crossers. For one tree with scene information, 22 of the 24 never-crossers raising these alarms stood at signalised locations (slightly more than the 82.2% of all test windows), with a median intention rating of 0.90. They were mostly intending pedestrians waiting to cross, often while the ego vehicle was itself waiting at a red light. Under our labels they are who errors, as they did not cross while the vehicle was present (𝐶 = 0), although their ratings suggest that many would have crossed after the footage ends (Section 3.4).

Figure 2 Separation of a crossing pedestrian’s windows from never-crosser windows, by time remaining until the onset (AUC, streaming test split, mean over seeds; the time bins have unequal widths). Anchored-trained models (dashed) show the reverse trend.

Table 5 When by distance to the crossing (streaming test split, post hoc, mean over seeds). Columns: time from the end of the negative window to the start of that pedestrian’s crossing; each cell ranks the same positives against the negatives in that column.

## Does a Timing Objective Help?

  - Onset Timing Under Censoring

An objective that models the time to the crossing, which we call the hazard objective, could use timing information that the binary label leaves unused. Starting from the ending of the observation window, the next 𝐿 = 96 frames were divided into

𝐾 = 24 bins of 𝑤 = 4 frames, and a hazard ℎ𝑘 was predicted for each bin: the probability that the next crossing starts in bin 𝑘 if it has not started before. The probability of an onset within 𝐻 then follows as

crossing starts after (s)

Model	1.07–2  2–3  3–5  5–10  ≥10 pooled

share of negatives (%)	6	6	10	19	59	–

𝑃(onset ≤ 𝐻) = 1 −

𝐻/𝑤−1

.

𝑘=0

(1 − ℎ𝑘) ,	(3)

trees, pedestrian only	0.603 0.684 0.770 0.776 0.855 0.805

trees + scene	0.635 0.734 0.805 0.809 0.897 0.845

neural net, pedestrian only 0.571 0.642 0.717 0.712 0.804 0.753

neural net + scene	0.612 0.679 0.728 0.727 0.853 0.791

at 0.809 to 0.918 AUC, whereas beyond 10 seconds the ranking fell to blind-guessing again between 0.470 and 0.584 AUC (Figure 2). Scene information and the oracles lifted the curve

which answers the question of Equation (1), so a hazard model can be scored exactly like a binary one. Targets 𝑧𝑘 and masks

𝑚𝑘 were derived from the frames to the next onset, the frames of future observed and whether the track contains a crossing. Bins after an observed onset and bins beyond the end of the footage were masked, and already-crossed windows were removed. The loss was the masked binary cross-entropy

while preserving the shape.

Far from the onset, the model scored a hard temporal negative window like a never-crosser window, therefore these windows

L = −

𝐾 −1

∑︁

𝑘=0

𝑚𝑘 h

𝑧𝑘 log ℎ𝑘 + (1 − 𝑧𝑘) log(1 − ℎ𝑘)

i.	(4)

are easily ranked below positive windows. Against the 59% of hard temporal negatives whose crossing starts ten seconds or more after the window, every streaming-trained model exceeded

0.80 (Table 5; post hoc), which keeps the pooled when score high. Against those whose crossing starts within two seconds, the models stayed between 0.571 and 0.635; scene information added 0.011 to 0.050 per column.

6.4 Why Per-Window Detection Lags

A better who score improved detection per pedestrian but not clearly per window, as expected from P3 (Section 3.4). Scene information followed this pattern in both learners: detection was better per pedestrian and inconclusive per window (both verdicts pre-registered). For the neural network, detection per window even fell at every budget, from 42.3 to 32.5% at 205 alarms per hour.

The analysis of this difference below is post hoc. With scene information, the 212 false-alarm windows of the trees allowed

Masked bins receive zero gradient, which allowed the 7 470 censored training windows to be used, whereas binary training has to drop them or label them 0. In the neural network, a hazard output replaced the binary one. For the trees, the hazard was fitted with the exact discrete-time likelihood, using one training row per observed bin and the bin index as an extra feature. At longer horizons, the bins were widened with the horizon, and these arms used three seeds.

  - Results

The hazard objective did not beat binary training. From Table 6, the comparison for the neural network was inconclusive at all five budgets. At 205 alarms per hour (counted per window), the hazard model detected 44.1 ± 4.7% of crossers and the binary

model 42.3 ± 3.2%. For the tree, binary training led by 3.5 to 5.7

points per window at every budget, but the interval excluded zero

at only 2 budgets, just short of the pre-registered bar. The result therefore favors binary training, but is ultimately inconclusive.

Table 6 Objectives on the streaming test split. Top: detection rate (%) per window (pw) or per pedestrian (pp) at 205 or 41 alarms per hour, and when, mean ± standard deviation over

seeds. ∗Its contrast with binary training is post hoc. Bottom:

trees at longer horizons, with detection per window at 205 alarms per hour and who. Binary training drops the censored windows or labels them 0; the hazard objective masks their unobserved bins.

Model	objective	pw 205	pw 41	pp 205	when

neural net binary	42.3 ± 3.2 22.9 ± 3.5 86.3 ± 1.3 0.753

neural net hazard	44.1 ± 4.7 17.4 ± 9.1 86.8 ± 1.5 0.766

trees	binary	51.4 ± 1.9 36.4 ± 2.4 86.0 ± 1.5 0.805

trees	hazard	46.9 ± 2.2 30.7 ± 2.9 84.0 ± 2.8 0.796

## Discussion

  - What This Means for Evaluation

An anchored score measures who alone. It can be high even when a model reverses the ordering of crossing times (Table 3), so it tells us little about how the model will behave on streaming data. Three changes to evaluation could be made with the existing annotations. Who and when should be reported separately. Evaluation should use streaming windows and a detection curve at fixed false-alarm budgets. False alarms should also be counted per pedestrian as well as per window, because a few waiting pedestrians can dominate the per-window count.

trees	hazard + censored trees	binary + censored as 0

54.9 ± 3.1 39.2 ± 1.5 86.0 ± 1.3 0.794

∗ 56.5 ± 2.5 38.5 ± 2.4 87.0 ± 2.0 0.798

  - What When Would Need

When remains the open problem. On PIE, none of the inputs we

pw 205	who

𝐻	drop zero hazard drop zero hazard

1.07 s 51.4 56.5  54.9  0.755 0.786 0.767

2.13 s 39.5 46.0  50.4  0.668 0.766 0.732

5.33 s 31.3 41.5  44.7  0.479 0.747 0.600

Censored windows helped the hazard objective. When the 7 470 censored windows were added, tree detection rate per window increased by +5.9 to +8.5 points (pre-registered). For binary training, labelling these windows 0 gave +2.1 to +5.2 points over leaving them out, although this post hoc contrast was inconclusive. The zero labels raised who AUC score from 0.755 to 0.786, suggesting that dropping these windows in our binary training lose some information.

Longer horizons. How censored windows are handled matters more at longer horizons, because a longer horizon leaves more windows without a full future. We therefore repeated the tree comparison at horizons of 2.13 and 5.33 s (Table 6, bottom). Against binary training that drops censored windows, the lead of the hazard objective grew with the horizon, from +2.3 to

+3.5 points at 1.07 s (inconclusive) to +10.1 to +16.4 points at 5.33 s (better), as predicted before the experiment. This is because dropping is biased: when a never-crosser’s footage ends, their last windows are dropped, while a crosser’s windows are kept whenever the crossing is seen. At 5.33 s, this left the dropping model ranking who below chance (0.479 AUC). Against binary training that labels censored windows 0, however, the hazard objective was inconclusive at every horizon. The hazard objective therefore beats dropping, not binary training as such.

Reading. These results agree with P2 (Section 3.4). The hazard objective adds supervision about when a crossing starts, which can only help if the inputs carry timing information that binary training leaves unused. Section 6 found little such information in our inputs, and the when scores of the two objectives were close (Table 6). The only clear gain of the hazard objective came from the censored windows, and it did not beat binary training that labels these windows 0 at any horizon.

added improved it by more than 0.043. Just beyond the horizon, for crossings that start within two seconds, it remained between 0.571 and 0.635 in a post hoc analysis (Table 5). The most persistent false alarms, examined post hoc as well, were who errors involving intending pedestrians at signalised crosswalks who might cross after the footage ends. Their crossing time may depend on information missing from the data, such as the signal controlling the pedestrian, gaps in oncoming traffic or interaction with the driver. Frame-level intent labels [15], longer footage after the observation window and waiting-time models [20, 21, 22] are natural next steps.

  - Limitations

Pre-registration. All studies were specified and time-stamped before their first result, with two amendments. Before any results were obtained, we loosened a check that the exported inputs reproduced the earlier network outputs, because those stored outputs had been computed in half precision. After the results were obtained, we found that the tree hazard readout had been stored in single precision. This rounded confident scores to exactly one, leaving three hazard arms with no threshold that could meet the tightest alarm budgets. Recomputing the readout in double precision changed two verdicts—from “binary better” to inconclusive—for the tree hazard comparisons against binary training, on the full input and on the input without ego-vehicle speed. Both versions are retained.

The pre-registered verdicts not discussed above were all incon-clusive per window: lights, traffic and layout individually against the pedestrian-only model; the scene with both oracles against that model, although this was better per pedestrian; the intent oracle beyond the scene; the detection contrasts in the waiting audit; hazard against binary training on an eighth, a quarter and half of the training pedestrians; focal loss and class-balanced weights against oversampling with streaming training; a split of the scene result by signalisation; and an earlier test combining an anchored model’s score with a streaming model’s score. Remov-ing ego-vehicle speed from the tree’s input was equivalent within 5 points per window (at 4 of the five budgets) and inconclusive per pedestrian. Adding the ego-future oracle to the scene was equivalent within 5 points, both per window and per pedestrian. Adding the censored windows to the hazard objective was better per window (Section 7.2) but inconclusive per pedestrian.

Accounting. The scene audit was inconclusive on its primary,

per-window measure. Its clear result was on the pre-registered secondary measure, counted per pedestrian. Our explanation for the difference between these measures in Section 6.4 is post hoc. Inputs. Scene and behaviour features came from PIE annota-tions, so they assume perfect perception of information a camera could see. Noisier, real-world perception would weaken the who result and strengthen the negative result for when. The light feature represents the largest light in view, which need not control the pedestrian. The finding that lights add little therefore applies to this feature rather than to right-of-way. Neither oracle is deployable, and the ego-future oracle is confounded. Its arms also reached the iteration cap while validation performance was still improving, so its effect may be understated.

Power and scope. The test split contains 205 crossing pedes-trians. Two seeds of one tree arm can differ by 6.8 points per window, and the tree comparisons resolved differences of about 5 points. We used the neural-network recipe despite its failure on a pre-registered training check: the networks overfit after the first few epochs, and we report the checkpoint with the best validation AUC. All results come from PIE, which was recorded in one city in daylight. Our anchored protocol is a reimplementa-tion, and its scores cannot be compared directly with published results. We chose the detection metric after obtaining the first window-level results. Earlier findings motivated the scene and waiting audits, but their questions were fixed before their own results were obtained. Excluding the 21 never-crossers who cross a different road changes detection per window by at most

2.8 points in a post hoc check.

## Conclusion

The streaming crossing label is the product of two factors, which are the probability that a pedestrian will cross at all, or who, and the probability that the crossing will begin within a certain horizon, or when. We highlight that, the event-anchored protocol measures only who. A model can therefore score well on anchored data while reversing the ordering of when, as we observed on PIE with both a neural network and boosted trees. We find from our experiments that, scene information that a camera or map could in principle provide raises who above a human intention rating in a post hoc comparison. When, by contrast, improves by at most 0.043 and remains low just beyond the horizon. Furthermore, with the same training windows, a hazard objective does no better than binary training, consistent with the decomposition. Based on these, we conclude that, evaluations should report who and when separately, as further progress in streaming crossing prediction is likely to depend on information about when pedestrians cross.

## References

- Rasouli, A., Kotseruba, I., Kunic, T., et al.: ‘PIE: A Large-Scale Dataset and Models for Pedestrian Intention Estimation and Trajectory Prediction’. Proc. IEEE/CVF Int. Conf. Computer Vision (ICCV), 2019, pp. 6262-6271

- Rasouli, A., Kotseruba, I., Tsotsos, J.K.: ‘Are They Going to Cross? A Benchmark Dataset and Baseline for Pedes-trian Crosswalk Behavior’. Proc. IEEE Int. Conf. Computer Vision Workshops (ICCVW), 2017, pp. 206-213

- Kotseruba, I., Rasouli, A., Tsotsos, J.K.: ‘Benchmark for Evaluating Pedestrian Action Prediction’. Proc. IEEE/CVF Winter Conf. Applications of Computer Vision (WACV), 2021, pp. 1258-1268

- De Geest, R., Gavves, E., Ghodrati, A., et al.: ‘Online Action Detection’. Proc. European Conf. Computer Vision (ECCV), 2016, pp. 269-284

- Shou, Z., Pan, J., Chan, J., et al.: ‘Online Detection of Action Start in Untrimmed, Streaming Videos’. Proc. European Conf. Computer Vision (ECCV), 2018, pp. 534-551

- Achaji, L., Moreau, J., Fouqueray, T., et al.: ‘Is Attention to Bounding Boxes All You Need for Pedestrian Action Prediction?’. Proc. IEEE Intelligent Vehicles Symp. (IV), 2022, pp. 895-902

- Gesnouin, J., Pechberti, S., Stanciulescu, B., et al.: ‘Assess-ing Cross-Dataset Generalization of Pedestrian Crossing Predictors’. Proc. IEEE Intelligent Vehicles Symp. (IV), 2022, pp. 419-426

- Azarmi, M., Rezaei, M., Wang, H., et al.: ‘Feature Impor-tance in Pedestrian Intention Prediction: A Context-Aware Review’, arXiv preprint arXiv:2409.07645, 2024

- Rasouli, A., Kotseruba, I.: ‘Diving Deeper Into Pedestrian Behavior Understanding: Intention Estimation, Action Prediction, and Event Risk Assessment’. Proc. IEEE Intelligent Vehicles Symp. (IV), 2024, pp. 1381-1388

- Kotseruba, I., Rasouli, A., Tsotsos, J.K.: ‘Do They Want to Cross? Understanding Pedestrian Intention for Behavior Prediction’. Proc. IEEE Intelligent Vehicles Symp. (IV), 2020, pp. 1688-1693

- Yao, Y., Atkins, E., Johnson-Roberson, M., et al.: ‘Cou-pling Intent and Action for Pedestrian Crossing Behavior Prediction’. Proc. Int. Joint Conf. Artificial Intelligence (IJCAI), 2021, pp. 1238-1244

- Keller, C.G., Gavrila, D.M.: ‘Will the Pedestrian Cross? A Study on Pedestrian Path Prediction’, IEEE Transactions on Intelligent Transportation Systems, 2014, 15, (2), pp.

494-506

- Kooij, J.F.P., Schneider, N., Flohr, F., et al.: ‘Context-Based Pedestrian Path Prediction’. Proc. European Conf.

Computer Vision (ECCV), 2014, pp. 618-633

- Guo, D., Mordan, T., Alahi, A.: ‘Pedestrian Stop and Go Forecasting with Hybrid Feature Fusion’. Proc. IEEE Int. Conf. Robotics and Automation (ICRA), 2022, pp. 940-947

- Jing, T., Chen, T., Tian, R., et al.: ‘PSI: A Benchmark for Human Interpretation and Response in Traffic Interac-tions’. Advances in Neural Information Processing Systems (NeurIPS), 2025, pp. 52742-52762

- Xie, C., Lin, C., Zheng, X., et al.: ‘GTransPDM: A Graph-Embedded Transformer with Positional Decoupling for Pedestrian Crossing Intention Prediction’, IEEE Signal Processing Letters, 2025, 32, pp. 2109-2113

- Cox, D.R.: ‘Regression Models and Life-Tables’, Journal of the Royal Statistical Society: Series B, 1972, 34, (2), pp. 187-202

- Farewell, V.T.: ‘The Use of Mixture Models for the Analy-sis of Survival Data with Long-Term Survivors’, Biomet-rics, 1982, 38, (4), pp. 1041-1046

- Kuk, A.Y.C., Chen, C.-H.: ‘A Mixture Model Combining Logistic Regression with Proportional Hazards Regression’,

Biometrika, 1992, 79, (3), pp. 531-541

- Hamed, M.M.: ‘Analysis of Pedestrians’ Behavior at Pedes-trian Crossings’, Safety Science, 2001, 38, (1), pp. 63-82

- Li, B.: ‘A Model of Pedestrians’ Intended Waiting Times for Street Crossings at Signalized Intersections’, Trans-portation Research Part B: Methodological, 2013, 51, pp. 17-28

- Yang, X., Abdel-Aty, M., Huan, M., et al.: ‘An Accelerated Failure Time Model for Investigating Pedestrian Crossing Behavior and Waiting Times at Signalized Intersections’,

Accident Analysis & Prevention, 2015, 82, pp. 154-162

- Kalatian, A., Farooq, B.: ‘DeepWait: Pedestrian Wait Time Estimation in Mixed Traffic Conditions Using Deep Survival Analysis’. Proc. IEEE Intelligent Transportation Systems Conf. (ITSC), 2019

- Yang, Z., Zeng, A., Yuan, C., et al.: ‘Effective Whole-body Pose Estimation with Two-stages Distillation’. Proc. IEEE/CVF Int. Conf. Computer Vision Workshops (IC-

CVW), 2023, pp. 4212-4222

- Friedman, J.H.: ‘Greedy Function Approximation: A Gradient Boosting Machine’, The Annals of Statistics, 2001, 29, (5), pp. 1189-1232

- Page, E.S.: ‘Continuous Inspection Schemes’, Biometrika, 1954, 41, (1/2), pp. 100-115

- Efron, B., Tibshirani, R.J.: ‘An Introduction to the Boot-strap’ (Chapman & Hall/CRC, 1993)

- Lakens, D.: ‘Equivalence Tests: A Practical Primer for 𝑡 Tests, Correlations, and Meta-Analyses’, Social Psycholog-ical and Personality Science, 2017, 8, (4), pp. 355-362

- Lin, T.-Y., Goyal, P., Girshick, R., et al.: ‘Focal Loss for Dense Object Detection’. Proc. IEEE Int. Conf. Computer Vision (ICCV), 2017, pp. 2980-2988

- Cui, Y., Jia, M., Lin, T.-Y., et al.: ‘Class-Balanced Loss Based on Effective Number of Samples’. Proc. IEEE/CVF Conf. Computer Vision and Pattern Recognition (CVPR), 2019, pp. 9268-9277